EXTRON  RGB 464xi安装接口

Extron RGB 460xi 系列是通用型模拟计算机视频接口，其水平频率范围为 15~150 kHz，RGB 视频带宽为 300 MHz（-3 dB），带有一个 15 针 HD 插座，用于将计算机连接至显示设备。它采用了 Extron ADSP?（高级数字同步处理）技术。ADSP 为同步信号提供了完全的数字处理，避免了将较差的同步处理方法用于数字投影机和平板显示设备时出现的图像扭曲及其它失真现象。

RGB 464xi

用于三单元墙盒的结构化安装接口，带视音频连接

主要特性 安装在三单元墙盒中. 包括复合视频直通连接器、RCA 至 BNC、S-视频直通连接器、4 针微型 DIN 插座对插座、有源立体声/单声道、非平衡至平衡立体声音频驱动器，以及用于网络、数据和电话的 RJ-45 连接器. 兼容 VGA-QXGA、Mac、Sun? 和 SGI? 计算机信号. 300 MHz（-3 dB）RGB 视频带宽. ADSP? – 高级数字同步处理. 型号; 产品说明; 产品编号. RGB 464xi; 黑色; 60-374-02. RGB 464xi; 白色; 60-374-03. Extron RGB 460xi 系列是通用型模拟计算机视频接口，其水平频率范围为 15~150 kHz，RGB 视频带宽为 300 MHz（-3 dB），带有一个 15 针 HD 插座，用于将计算机连接至显示设备。它采用了 Extron ADSP?（高级数字同步处理）技术。ADSP 为同步信号提供了完全的数字处理，避免了将较差的同步处理方法用于数字投影机和平板显示设备时出现的图像扭曲及其它失真现象。

| Video |  |
| --- | --- |
| Gain | Unity (0.7 V), 15% with 3 dB peaking (0.8 V), 30% with 6 dB peaking (0.9 V) |
| Bandwidth | 300 MHz (-3 dB) |
| Video input and loop through |  |
| Number/signal type | 1 analog RGBHV, RGBS, RGsB |
| Connectors | 1 female 15-pin HD buffered input connector 1 female 15-pin HD local monitor loop through connector |
| Nominal level | 0.7 Vp-p for RGB |
| Minimum/maximum levels | Analog: 0.3 V to 1.45 Vp-p with no offset at unity gain |
| Impedance | 75 ohms or Hi Z, switchable (set to 75 ohms if no local monitor is connected) |
| Horizontal frequency | 15 kHz to 150 kHz (optimum: 15 kHz to 62 kHz) |
| Vertical frequency | 30 Hz to 170 Hz |
| Return loss | <-30 dB @ 5 MHz |
| Video output |  |
| Number/signal type | 1 analog RGBHV, RGBS, RGsB |
| Connectors | 5 female BNC on 2" to 2.5" cables |
| Nominal level | 0.7 Vp-p for RGB |
| Minimum/maximum levels | Analog: 0.7 V to 0.9 Vp-p (switch selectable) with peaking and 0.7 Vp-p input |
| Impedance | 75 ohms |
| Return loss | -30 dB @ 5 MHz |
| Sync |  |
| Input type | Autodetect RGBHV, RGBS, RGsB (accepts RGsB but does not strip from video) |
| Output type | RGBHV: at all times RGBS: switch selectable RGsB: switch selectable or pass-through |
| Input level | 2.0 V to 5.5 Vp-p with +0.2 VDC offset max. |
| Output level | 4.0 V to 5.0 Vp-p, unterminated |
| Input impedance | 510 ohms |
| Output impedance | 75 ohms |
| Max. propagation delay | 52 ns |
| Max. rise/fall time | 2.5 ns |
| Polarity | Positive or negative |
| Audio — all models |  |
| Gain | Unbalanced output: 0 dB; balanced output: +6 dB |
| Response | 20 Hz to 20 kHz, ±0.5 dB |
| THD + Noise | 0.03% @1 kHz, 0.3% @ 20 kHz at nominal level |
| S/N | >90 dB at maximum output (14 dBu), balanced (unweighted) |
| Stereo channel separation | >90 dB @ 1 kHz to 20 kHz |
| Audio input — RGB 464xi |  |
| Number/signal type | 2 stereo, unbalanced, 20 Hz to 20 kHz |
| Connectors | Ch. 1: 3.5 mm female stereo jack (2 channel), unbalanced; tip (L), ring (R), sleeve (ground) Ch. 2: 2 RCA female jacks |
| Impedance | >5k ohms unbalanced, DC coupled |
| Nominal level | -10 dBV (316 mV) |
| Maximum level | +8.5 dBu (unbalanced) at 1% THD+N |
| Audio input — RGB 460xi, RGB 460xi EC, RGB 460xi FSR, RGB 468Mxi, RGB 468xi, RGB 468xi AKM |  |
| Number/signal type | 1 stereo, unbalanced, 20 Hz to 20 kHz |
| Connectors | 3.5 mm female stereo jack (2 channel), unbalanced; tip (L), ring (R), sleeve (ground) |
| Impedance | >5k ohms unbalanced, DC coupled |
| Nominal level | -10 dBV (316 mV) |
| Maximum level | +8.5 dBu (unbalanced) at 1% THD+N |
| Audio output — RGB 464xi |  |
| Number/signal type | 2 buffered, stereo (2 channel), balanced/unbalanced |
| Connectors | (2) 3.5 mm captive screw connectors, 5 pole |
| Impedance | 50 ohms unbalanced, 100 ohms balanced |
| Gain error | ±0.1 dB channel to channel |
| Maximum level (600 ohm) | +14 dBm, balanced at 1% THD+N |
| Audio output — RGB 460xi, RGB 460xi EC, RGB 460xi FSR, RGB 468Mxi, RGB 468xi, RGB 468xi AKM |  |
| Number/signal type | 1 buffered, stereo (2 channel), balanced/unbalanced |
| Connectors | 3.5 mm captive screw connector, 5 pole |
| Impedance | 50 ohms unbalanced,100 ohms balanced |
| Gain error | ±0.1 dB channel to channel |
| Maximum level (600 ohm) | +14 dBm, balanced at 1% THD+N |
| NOTE; 0 dBu = 0.775 V, 0 dBV = 1 V, 0 dBV ≈ 2 dBu.. |
| General |  |
| Power | Supplied by an included 12 VDC, 1 A external power supply |
| External power supply | 100 VAC to 240 VAC, 50-60 Hz, external; to 9 to 12 VDC, 0.2 A, regulated |
| Power input requirements | 12 VDC, 0.2 A (minimum) |
| Temperature/humidity | Storage: -40 to 158 °F (-40 to +70 °C) / 10% to 90%, noncondensing Operating: +32° to +122 °F (0 to +50 °C) / 10% to 90%, noncondensing |
| Cooling | Convection, no vents |
| Mounting |  |
| Floor box mount |  |
| RGB 460/464/468xi SC models | Steel City AFM floor box mountable |
| RGB 460xi MK | MK box mountable |
| RGB 468xi AKM | Ackerman UK floor box mountable |
| All other models | Wall or furniture mountable |
| Enclosure type | Metal |
| Enclosure dimensions |  |
| RGB 460xi faceplate | 4.5" H x 4.6" W x 0.1" D (2 gang) (11.4 cm H x 11.7 cm W x 0.3 cm D) |
| RGB 460xi MK faceplate | 3.4" H x 5.7" W x 0.1" D (8.6 cm H x 14.5 cm W x 0.3 cm D) |
| RGB 460xi FSR faceplate | 4.5" H x 3.5" W x 0.1" D (11.4 cm H x 9.0 cm W x 0.3 cm D) |
| RGB 464xi/RGB 468Mxi faceplate | 4.5" H x 6.4" W x 0.1" D (3 gang) (11.4 cm H x 16.3 cm W x 0.3 cm D) |
| RGB 468xi faceplate | 4.5" H x 8.3" W x 0.1" D (4 gang) (11.4 cm H x 21.2 cm W x 0.3 cm D) |
| RGB 468xi AKM faceplate | 3.0" H x 8.1" W x 0.6" D (7.6 cm H x 20.6 cm W x 1.5 cm D) |
| RGB 460xi EC faceplates | 3.2" H x 4.4" W x 0.1" D (8.0 cm H x 11.2 cm W x 0.3 cm D) |
| RGB 460xi SC-AFM2 faceplate | 2.9" H x 3.5" W x 0.1" D (7.4 cm H x 8.9 cm W x 0.3 cm D) |
| RGB 464xi SC-AFM6 faceplate | 2.9" H x 8.9" W x 0.1" D (7.4 cm H x 22.6 cm W x 0.3 cm D) |
| Interface enclosure for all models | 2.7" H x 3.3" W x 1.0" D (6.9 cm H x 8.4 cm W x 2.8 cm D) (Depth excludes front panel connectors and controls.) |
| Enclosure dimensions |  |
| SC-AFM8 adapter plate | 4.4" x H x 9.4" W x 0.1" D (11.2 cm H x 23.9 cm W x 0.3 cm D) |
| Product weight |  |
| RGB 460xi SC-AFM2, RGB 460xi FSR | 0.5 lbs (0.3 kg) |
| RGB 464xi SC-AFM6 | 0.8 lbs (0.4 kg) |
| SC-AFM8 | 0.6 lbs (0.3 kg) |
| RGB 460xi | 0.5 lbs (0.2 kg) |
| RGB 460xi MK | 0.5 lbs (0.2 kg) |
| RGB 464xi, RGB 468xi, RGB 468 Mxi | 0.7 lbs (0.3 kg) |
| EC models | 0.7 lbs (0.3 kg) |
| RGB 468xi AKM | 0.8 lbs (0.4 kg) |
| Shipping weight |  |
| RGB 468xi AKM | 2 lbs (1 kg) |
| All other models | 3 lbs (2 kg) |
| Vibration | ISTA 1A in carton (International Safe Transit Association) |
| Regulatory compliance |  |
| Safety | CE, c-UL, UL |
| EMI/EMC | CE, C-tick, FCC Class A, ICES, VCCI |
| MTBF | 30,000 hours |
| Warranty | 3 years parts and labor |